# Design Note: Robot Vacuum Control

# How I built it

I built this with an AI code assistant (Claude Code), one stage at a time. For each stage I wrote a prompt, reviewed the code, tested it on the simulator and fed any problems back into the next prompt.

# Roadmap

- UI. Laid out the window in .ui files first, so every later stage had a place to plug in.

- Communication. Connected to the simulator over ZeroMQ: RPC, commands and the 50 Hz state stream on its own thread. I drive with wheel commands instead of goto, which ignores obstacles and stops a few cm off. The cost: commands must go out at 20 Hz to beat the 250 ms watchdog.

- Map. Loaded and drew the occupancy map, and checked its coordinates against the debug viewer.

- Movement. Added manual driving. All commands go through one class (Navigator) so they never conflict. A simple filter combines wheel odometry with the noisy pose.

- Path planning. A* on a cost map that keeps the robot away from walls, then shortened into straight segments. If there is no path, it retries with a tighter margin.

- Reaching goals. Follows the path, slows down in tight spaces, and turns to the goal's yaw. It calls mark_goal only when within 3 cm / 3°, since only the first mark counts.

- Collisions. On a bump, it marks the obstacle, backs off and re-plans.

- Recording. Saves every RGB and depth frame with timestamps, robot pose and camera calibration, plus preview videos.

- Finishing. Logging, CMake build and README.

# Architecture

┌──────────── GUI thread ─────────────────────────────┐
 5557 ──► StateSubscriber ──signal──► MainWindow ──► PoseEstimator
 (thread)                              │                 │ filtered pose
                                       ▼                 ▼
                                    MapView ◄──signals── Navigator (20 Hz QTimer)
                                                         │  CostMap + PathPlanner
 5555 ◄── SimClient (RPC, mutex) ◄───────────────────────┤
 5556 ◄── SimClient (PUSH wheels) ◄──────────────────────┘
 5557/5558 ──► CameraRecorder (own thread, own sockets) ──► recording folder

The core library has no UI code. State and recording run on their own threads.

# Limitations and next steps

Obstacles are found only by bumping into them. Next: use the depth camera to see them first.

A busy GUI can delay control. Next: move control to its own thread.

No automated tests. Next: unit tests and smoother paths and pose filter.